【实验目的】

用averaging kernel, Gaussian kernel, Bilateral filter, and Midean filter四种方法对含有高斯噪声和椒盐噪声的图像进行去噪。

【运行结果】

实验中我用到的误差统计方法是，PSNR也统计过，但是可视化不明显，所以用范数进行统计。

error=100×||I'-I||F||I||F

在这里统计一下数据（效果最佳从上至下）：

| 【Method】 | 【Error】 |
| --- | --- |
| Median filter | 6.6786 |
| Gaussian kernel σs=10 | 17.5993 |
| Bilateral filter σs=10、σr=200 | 18.0503 |
| Bilateral filter σs,r自适应 | 20.1723 |
| Averaging kernel | 23.5543 |
| Bilateral filter σs=10、σr=0.1 | 61.5071 |

【分析一波】

- 中值滤波对椒盐噪声的原图像处理要明显好于其他几个方法，猜测原图像保留的像素点较多，再一个在中值选择像素点的过程中，留下了更多原图像像素点，所以效果非常好。

- 对于第二中的高斯核、第三与最后的双边滤波，采取的σs都是10，对于Gaussian kernel，可以看作是σr=∞，故随着σr的不断减小，误差值是越来越大的，为了确保我说的并不带有主观性，我找到了CSDN上其他人写的代码，并加以实现，得到以下结果：

这里显示的σr依旧是一个很大的值，并且直观效果也并不好。

- 对于第四个双边滤波，σs与σr自适应，采取的方法是：每次核化都用当前卷积核对应图像区块的方差作为σs与σr的取值，所以得到了一个数值与直观上都不错的结果。

- 在效果比较好的这些方法中，Averaging kernel效果也不错误差值也是在20左右，只是从精确度上来说，就没有前几者的高了。

【疑问】

对于Bilateral kernel来说，若按照课件上给的σs与σr给一个特定值，σs=16，σr=0.1，实验结果运行并不是很好，反而给σr一个十分大的值，即几乎可以去掉Bilateral kernel的权值的后半项（I’-I项），是高斯核的近似，那么我们得到一个比较好的效果图，所以σs与σr到底应该怎么取值？本实验中采取二者自适应的取法效果比较好。

【源代码】

clear

clc

f=imread('riven.jpg');

g=rgb2gray(f);

g1=imnoise(g,'gaussian',0,0.1);

g2=imnoise(g,'salt & pepper',0.2);

subplot(331);imshow(g),title('【原图】灰度');

[m,n]=size(g);

normg=norm(double(g),'fro');

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

X2=zeros(m+2,n+2);

for i=1:m

for j=1:n

X2(i+1,j+1)=g1(i,j);

end

end

subplot(332);imshow(uint8(X2)),title('【高斯噪】padding');

X3=zeros(m+4,n+4);

for i=1:m

for j=1:n

X3(i+2,j+2)=g1(i,j);

end

end%用了两种大小的高斯噪声padding

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

X4=zeros(m+4,n+4);

for i=1:m

for j=1:n

X4(i+2,j+2)=g2(i,j);

end

end

X=zeros(m+2,n+2);

for i=1:m

for j=1:n

X(i+1,j+1)=g2(i,j);

end

end%用了两种大小的椒盐噪声padding

subplot(333);imshow(uint8(X)),title('【椒盐噪】padding');

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

averaging=[1/9,1/9,1/9;1/9,1/9,1/9;1/9,1/9,1/9];%averaging filter，1/9各元素

Y=zeros(m,n);

for i=1:m

for j=1:n

sum=0;

for k=1:3

for l=1:3

sum=sum+averaging(k,l)*X2(i+k-1,j+l-1);%X2是（m+2，n+2）大小

end

end

Y(i,j)=sum;

end

end

error=100*norm(Y-double(g),'fro')/normg;

subplot(334);imshow(uint8(Y)),title({'【averaging kernel】', ['norm_F error= ', num2str(error)] })

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

Gaussian=zeros(5,5);%σ取2，高斯核

sumGaussian=0;%系数的和

for i=1:5

for j=1:5

Gaussian(i,j)=(1/sqrt(2*pi)*exp(-(((i-3)^2+(j-3)^2)/2)));

end

end

for i=1:5

for j=1:5

sumGaussian=sumGaussian+Gaussian(i,j);

end

end

Gaussian=Gaussian/sumGaussian;%给高斯核的系数做一个归一化处理

Z=zeros(m,n);

for i=1:m

for j=1:n

sum=0;

for k=1:5

for l=1:5

sum=sum+Gaussian(k,l)*X3(i+k-1,j+l-1);

end

end

Z(i,j)=sum;

end

end

error=100*norm(Z-double(g),'fro')/normg;

subplot(335);imshow(uint8(Z)),title({'【Gaussian kernel】', ['norm_F error= ', num2str(error)] })

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

B1=zeros(5,5);%这是双边滤波系数的第一项

s1=zeros(5,5);%开始求一个距离权值的方差

for i=1:5

for j=1:5

s1(i,j)=sqrt((i-2)^2+(j-2)^2);

end

end

s=(std2(s1))^2;

for i=1:5

for j=1:5

B1(i,j)=exp(-((i-3)^2+(j-3)^2)/(2*s^2));

end

end

Z=zeros(m,n);

for i=1:m

for j=1:n

sum=0;%一次卷积的加权平均

sumB=0;%给系数做归一化处理

var1=zeros(5,5);

var1(1:5,1:5)=X3(i-1+1:i-1+5,j-1+1:j-1+5)-X3(i-1+3,j-1+3);

r=(std2(var1))^2;

for k=1:5

for l=1:5

sumB=sumB+B1(k,l)*exp(-(((X3(i-1+k,j-1+l)-X3(i-1+3,j-1+3))^2)/(2*(r^2))));

end

end

for k=1:5

for l=1:5

sum=sum+(B1(k,l)/sumB)*X3(i+k-1,j+l-1)*exp(-(((X3(i-1+k,j-1+l)-X3(i-1+3,j-1+3))^2)/(2*(r^2))));

end

end

Z(i,j)=sum;

end

end

error=100*norm(Z-double(g),'fro')/normg;

subplot(339);imshow(uint8(Z)),title({'【Bilateral filter σs,r自适应】', ['norm_F error= ', num2str(error)] })

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

B1=zeros(5,5);%这是双边滤波系数的第一项

s=10;r=200;

for i=1:5

for j=1:5

B1(i,j)=exp(-((i-3)^2+(j-3)^2)/(2*s^2));

end

end%没有问题的值

Z=zeros(m,n);

for i=1:m

for j=1:n

sum=0;%卷积运算的和

sumB=0;%权值求和，归一化处理

for k=1:5

for l=1:5

sumB=sumB+B1(k,l)*exp(-(((X3(i-1+k,j-1+l)-X3(i-1+3,j-1+3))^2)/(2*(r^2))));

end

end

for k=1:5

for l=1:5

sum=sum+(B1(k,l)/sumB)*X3(i+k-1,j+l-1)*exp(-(((X3(i-1+k,j-1+l)-X3(i-1+3,j-1+3))^2)/(2*(r^2))));

end

end

Z(i,j)=sum;

end

end

error=100*norm(Z-double(g),'fro')/normg;

subplot(337);imshow(uint8(Z)),title({'【Bilateral filter，σs=10，σr=200】', ['norm_F error= ', num2str(error)] })

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

B1=zeros(5,5);%这是双边滤波系数的第一项

s=16;r=0.1;

for i=1:5

for j=1:5

B1(i,j)=exp(-((i-3)^2+(j-3)^2)/(2*s^2));

end

end%没有问题的值

Z=zeros(m,n);

for i=1:m

for j=1:n

sum=0;%卷积运算的和

sumB=0;%权值求和，归一化处理

for k=1:5

for l=1:5

sumB=sumB+B1(k,l)*exp(-(((X3(i-1+k,j-1+l)-X3(i-1+3,j-1+3))^2)/(2*(r^2))));

end

end

for k=1:5

for l=1:5

sum=sum+(B1(k,l)/sumB)*X3(i+k-1,j+l-1)*exp(-(((X3(i-1+k,j-1+l)-X3(i-1+3,j-1+3))^2)/(2*(r^2))));

end

end

Z(i,j)=sum;

end

end

error=100*norm(Z-double(g),'fro')/normg;

subplot(338);imshow(uint8(Z)),title({'【Bilateral filter，σs=16，σr=0.1】', ['norm_F error= ', num2str(error)] })

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

Z=zeros(m,n);

for i=1:m

for j=1:n

kernel=zeros(3,3);

for k=1:3

for l=1:3

kernel(k,l)=X(i-1+k,j-1+l);

end

end

kernelmedian=median(kernel,'all');

Z(i,j)=kernelmedian;

end

end

error=100*norm(Z-double(g),'fro')/normg;

subplot(336);imshow(uint8(Z)),title({'【Median filter】', ['norm_F error= ', num2str(error)] })

【CSDN代码】

close all;

clc;

img=imread('riven.jpg');

img=rgb2gray(img);

[m,n]=size(img);

subplot(131);imshow(img);title('原图');

img1=imnoise(g,'Gaussian',0,0.0246);

subplot(132);imshow(img1);title('加上高斯噪声');

r=10;%模板半径

imgn=zeros(m+2*r+1,n+2*r+1);

imgn(r+1:m+r,r+1:n+r)=img1;

imgn(1:r,r+1:n+r)=img1(1:r,1:n);%扩展上边界

imgn(1:m+r,n+r+1:n+2*r+1)=imgn(1:m+r,n:n+r);%扩展右边界

imgn(m+r+1:m+2*r+1,r+1:n+2*r+1)=imgn(m:m+r,r+1:n+2*r+1);%扩展下边界

imgn(1:m+2*r+1,1:r)=imgn(1:m+2*r+1,r+1:2*r);%扩展左边界

sigma_d=2;

sigma_r=0.1;

[x,y] = meshgrid(-r:r,-r:r);

w1=exp(-(x.^2+y.^2)/(2*sigma_d^2));%以距离作为自变量高斯滤波器

%h=waitbar(0,'wait...');%初始化waitbar

for i=r+1:m+r

for j=r+1:n+r

w2=exp(-(imgn(i-r:i+r,j-r:j+r)-imgn(i,j)).^2/(2*sigma_r^2));

%以周围和当前像素灰度差值作为自变量的高斯滤波器

w=w1.*w2;

s=imgn(i-r:i+r,j-r:j+r).*w;

sumw=0;

for k=1:2*r+1

for l=1:2*r+1

sumw=sumw+w(k,l);

end

end

sums=0;

for k=1:2*r+1

for l=1:2*r+1

sums=sums+s(k,l);

end

end

imgn(i,j)=sums/sumw;

end

%waitbar(i/m);

end

%close(h)

img2=imgn(r+1:m+r,r+1:n+r);

%figure;

subplot(133);imshow(uint8(img2));title('网上代码得到的双边滤波结果');